| A:1PC | B:1PC |  | C:1PC | D:1PC | E:2PCS | F:2PCS | G:1PC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H:2PCS | I:8PCS | J:4PCS | K:4PCS | L:4PCS | M:1PC | N:1PC | O:4PCS | P:2PC |
| M6 | M6*12 | M8 | M8 NUT | TAPPINGSCREW | 4MM ALLEN KEY | 5MM ALLEN KEY | M6*16 | M4*40 |

SMLFT (S650, S1000, S1300)

TV LIFT INSTRUCTIONS

Step 1.                         Step 2.                    Step 3.

Step 4.

Height Memory Presets

Stop

Up(automatic)

Down(automatic)

Set memory

Memory

Set

Height Presets

Up

Down

Warning: Please measure all relevant components before installation to avoid any interference or collisions

## Reset

Reset the TV lift unit after installation using the following steps:

- Press the down button on the control panel for 5 seconds until the screen shows ‘RST’.

- Keep pressing the down button until the column has gone down to its lowest height and then bring it back up a tiny amount. Please do not release the down button until the ‘RST’ disappears off the display.

When the ‘RST’ has disappeared, the setup process has finished.

Note: if the screen shows ‘RST’ during normal operation, please follow step 2 to reset the lift.

## Set memory height

Once the TV Lift unit is installed we suggest setting the desired height using the memory functions.

- Operate the TV lift to the desired height that you want to set the memory function to.

- Press button ‘S’ on the handset or ‘M’ on the remote controller for 3 seconds till the screen shows ‘SET’

- Press a number (e.g. 1 or 2 on the remote or 1,2,3 or 4 on the lift control panel) where you want to save the preset. (within 3 seconds after the screen shows ’SET’)

- The preset height is stored when the screen shows the ’s-number’. (e.g. ‘ S-1’)

## Error

If the screen shows ‘ER1’, the cables are probably not connected correctly. Please check and reconnect all cables.

After re-connecting the cables, press the down button for 3 seconds and follow the above reset steps.

## Pairing remote controller

If the remote control is not working, please follow the below steps:

- The pairing window time is 3 seconds after the system is powered on. The end of the pairing window period is identified by a single beep

- During the 3 second pairing window, press & hold the "up" button of the remote control, wait for 4 long beeps and 3 short beeps to signal that pairing has been completed

- After the pairing completion confirmation beeps, release the "up" button

- The lift control system will now recognize the remote control.

Note: If during normal operation the lift encounters a sudden change in load, the controller will retract the lift in order to prevent damage to the unit.